EX NO: 7.)  DATE: CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

CODE:

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2;               % Message amplitude (V)

fm = 500;             % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1;               % Carrier amplitude (V)

fc = 10000;           % Carrier frequency (Hz)

%% FM Parameters

kf = 250;             % Frequency sensitivity (Hz/V)

fs = 100000;          % Sampling frequency (Hz)

T  = 0.01;            % Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am * cos(2*pi*fm*t);

%% Frequency Deviation & Modulation Index

delta_f = kf * Am;     % Peak frequency deviation (Hz)

beta    = delta_f / fm;

%% FM Signal Generation

fm_signal = Ac * cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Command Window Metrics

fprintf('Carrier Frequency (fc)     = %.0f Hz\n', fc);

fprintf('Message Frequency (fm)     = %.0f Hz\n', fm);

fprintf('Message Amplitude (Am)     = %.0f V\n', Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n', kf);

fprintf('\nFrequency Deviation (Δf)   = %.2f Hz\n', delta_f);

fprintf('Modulation Index (Beta)    = %.2f\n', beta);

%% Verification

beta_verify = delta_f / fm;

fprintf('Verified Modulation Index  = %.2f\n', beta_verify);

%% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting Results

figure;

subplot(2,1,1)

plot(t, m, 'LineWidth', 1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

grid on;

subplot(2,1,2)

plot(t, fm_signal, 'LineWidth', 1);

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

grid on;

WAVEFORM:

OUTPUT: