EXP. NO: 17

PERFORMANCE EVALUATION OF DIGITAL COMMUNICATION SYSTEMS USING EYE DIAGRAM

ANALYSIS UNDER ADDITIVE WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

MATHLAB CODE:

clc;

clear;

close all;

%% Parameters

N = 100; % Number of bits

sps = 20; % Samples per symbol

SNR = 10; % Signal-to-Noise Ratio (dB)

%% Generate Random Binary Data

data = randi([0 1],1,N);

%% NRZ Encoding (0 -> 0, 1 -> 1)

tx_signal = repelem(data,sps);

%% Pass Signal through AWGN Channel

rx_signal = awgn(tx_signal,SNR,'measured');

%% Time Axis

t = (0:length(tx_signal)-1)/sps;

%% Plot Results

figure;

% Original Binary Data

subplot(4,1,1)

stairs(data,'LineWidth',1.5)

grid on

title('Original Binary Data')

xlabel('Bit Number')

ylabel('Bit')

ylim([-0.2 1.2])

% Transmitted NRZ Signal

subplot(4,1,2)

plot(t,tx_signal,'LineWidth',1.5)

grid on

title('Transmitted NRZ Signal')

xlabel('Time')

ylabel('Amplitude')

ylim([-0.5 1.5])

% Received Signal after AWGN

subplot(4,1,3)

plot(t,rx_signal,'r')

grid on

title(['Received Signal with AWGN (SNR = ',num2str(SNR),' dB)'])

xlabel('Time')

ylabel('Amplitude')

% Eye Diagram

subplot(4,1,4)

eyediagram(rx_signal,2*sps)

title('Eye Diagram of Received Signal')

WAVEFORM:

OUTPUT: