EX NO: 2.)   CHARACTERISTICS STUDY OF SIDE BAND POWER AND FREQUENCIES IN AM USING MATLAB

CODE:

clc;

clear;

close all;

% Parameters

Am = 1;               % Message amplitude

fm = 500;             % Message frequency

Ac = 5;               % Carrier amplitude

fc = 5000;            % Carrier frequency

fs = 20 * fc;         % Sampling frequency

t  = 0:1/fs:4/fm;     % Time vector (4 cycles of message)

% Signals

m  = Am * sin(2*pi*fm*t);

c  = Ac * sin(2*pi*fc*t);

mu = Am / Ac;         % Modulation index

% AM Modulation

am = Ac * (1 + mu*sin(2*pi*fm*t)) .* sin(2*pi*fc*t);

% Envelope Demodulation

dem = abs(hilbert(am));

dem = dem - mean(dem); % Remove DC offset

% Frequency Domain Analysis

N      = length(am);

f      = linspace(-fs/2, fs/2, N);

AM_FFT = abs(fftshift(fft(am))) / N;

% Frequency Specs & Power Calculations

USB = fc + fm;

LSB = fc - fm;

Pc  = (Ac^2) / 2;

Psb = (mu^2 * Pc) / 4;

% Display Visual Results

figure('Name', 'AM Modulation', 'NumberTitle', 'off');

subplot(3,2,1)

plot(t, m)

title('Message')

grid on

subplot(3,2,2)

plot(t, c)

title('Carrier')

grid on

subplot(3,2,3)

plot(t, am)

title('AM Signal')

grid on

subplot(3,2,4)

plot(t, dem)

title('Demodulated')

grid on

subplot(3,2,5)

plot(f, AM_FFT)

xlim([fc-3*fm, fc+3*fm])

title('Sidebands')

grid on

subplot(3,2,6)

bar([Pc, Psb, Psb])

set(gca, 'XTickLabel', {'Carrier', 'USB', 'LSB'})

title('Power')

grid on

% Display Command Window Metrics

fprintf('μ=%.2f | USB=%d Hz | LSB=%d Hz | Pc=%.2f W | Psb=%.3f W\n', mu, USB, LSB, Pc, Psb);

WAVEFORM:

OUTPUT: